 Speakers

General Construction tips

- Make sure the cabinet is airtight.  A leaky cabinet will not perform well, and will lack bass.

- Brace well.  In larger cabinets, 1” x 2” braces at least every 18” are recommended.  A panel that vibrates will make the system perform poorly, and it will usually sound “throaty,” “boomy” or “nasal.”  Remember, you can’t overbrace!

- Use R11 or R13 insulation, covering the bottom, back and side nearest the port.

- For tower speakers and cornerhorns, hang a sheet of insulation on each brace, spanning the cabinet cross-section from side to side and front to back.  Bass will pass right through it, but midrange and any unwanted standing waves will be greatly reduced.

- For speakers with H290 horns and 1” exit compression horns, there are some extra “helpful hints” below:

Eminence

If using the H290 horn with two-hole mounts, no special steps are required.  This is recommended for use with the Eminence MD 2001 and PSD2002.

JBL

If using the H290 horn with three-hole mounts, remove the throat plate from the H290 horn flare and insert the mounting bolts before assembly.  Then, the mounting plate should be re-attached to the horn flare, and the compression driver attached with an open-end wrench.  This is the recommended procedure for the JBL 2426.

www.PiSpeakers.com