 Speakers

Crossover used by model number

| Model | Woofer | Tweeter | Xover | R1 | R2 | C1 | R3 | C5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Thermionic 3 | Alpha 12 | PSD2002 | 1K6a012dB | 25  | 16  | 0.47F | 8  | F |
| Thermionic 4 | Alpha 15 | PSD2002 | 1K6a012dB | 25  | 16  | 0.47F | 8  | F |
| Thermionic 6 | Alpha 12 | PSD2002 | 1K6a010dB | 16  | 16  | 0.47F | 8  | F |
| Thermionic 7 | Alpha 15 | PSD2002 | 1K6a010dB | 16  | 16  | 0.47F | 8  | F |
| Theater 3 | Delta 12LF | PSD2002 | 1K6a012dB | 25  | 16  | 0.47F | 8  | F |
| Theater 4 | Delta 15 | PSD2002 | 1K6a010dB | 16  | 16  | 0.47F | 8  | F |
| Theater 6 | Delta 12LF | PSD2002 | 1K6a010dB | 16  | 16  | 0.47F | 8  | F |
| Theater 7 | Delta 15 | PSD2002 | 1K6a008dB | 12  | 20  | 0.47F | 8  | F |
| Stage 3 | Omega 12 | PSD2002 | 1K6a012dB | 25  | 16  | 0.47F | 8  | F |
| Stage 4 | Omega 15 | PSD2002 | 1K6a012dB | 25  | 16  | 0.47F | 8  | F |
| Stage 4-18 | Omega 18 | PSD3003 | 800a015dB | 30  | 12  | F | 8  | F |
| Stage 6 | Omega 12 | PSD2002 | 1K6a010dB | 16  | 16  | 0.47F | 8  | F |
| Stage 7 | Omega 15 | PSD2002 | 1K6a010dB | 16  | 16  | 0.47F | 8  | F |
| Stage 7-18 | Omega 18 | PSD3003 | 800a012dB | 20  | 14  | F | 8  | F |
| Premium 4 | Kilomax 15 | PSD2002 | 1K6a012dB | 25  | 16  | 0.47F | 8  | F |
| Premium 4-18 | Kilomax 18 | PSD3003 | 500a015dB | 30  | 8  | F | 8  | F |
| Premium 7 | Kilomax 15 | PSD2002 | 1K6a010dB | 16  | 16  | 0.47F | 8  | F |
| Premium 7-18 | Kilomax 18 | PSD3003 | 500a012dB | 20  | 10  | F | 8  | F |
| Pro 3 (E) | JBL2206 | PSD2002 | 1K6a014dB | 30  | 14  | 0.33F | 8  | F |
| Pro 3 (J) | JBL2206 | JBL2426 | 1K6a014dB | 30  | 14  | 0.33F | 8  | F |
| Pro 4 (E) | JBL2226 | PSD2002 | 1K6aL12dB | 25  | 16  | 0.47F | NA | NA |
| Pro 4 (J) | JBL2226 | JBL2426 | 1K6aL12dB | 25  | 16  | 0.47F | NA | NA |
| Pro 4-18 (E) | JBL2242 | PSD3003 | 800a015dB | 30  | 12  | F | 8  | F |
| Pro 4-18 (J) | JBL2242 | JBL2446 | 800a015dB | 30  | 12  | F | 8  | F |
| Pro 6 (E) | JBL2206 | PSD2002 | 1K6a012dB | 25  | 16  | F | 8  | F |
| Pro 6 (J) | JBL2206 | JBL2426 | 1K6a012dB | 25  | 16  | 0.47F | 8  | F |
| Pro 7 (E) | JBL2226 | PSD2002 | 1K6aL10dB | 16  | 16  | 0.47F | NA | NA |
| Pro 7 (J) | JBL2226 | JBL2426 | 1K6aL10dB | 16  | 16  | 0.47F | NA | NA |
| Pro 7-18 (E) | JBL2242 | PSD3003 | 800a012dB | 20  | 14  | F | 8  | F |
| Pro 7-18 (J) | JBL2242 | JBL2446 | 800a012dB | 20  | 14  | F | 8  | F |
| Pro 10 | JBL2226 | PSD2002 | ten π xover | 25  | 20  | 0.47F | NA | NA |

 Speakers

Compensation component values by crossover

Crossover 1K6

| Part Number | Xover | HF atten | R1 | R2 | C1 |
| --- | --- | --- | --- | --- | --- |
| 1K6ax06dB | 1600 | 6dB | 12  | 30  | 0.47F |
| 1K6ax08dB | 1600 | 8dB | 12  | 20  | 0.47F |
| 1K6ax10dB | 1600 | 10dB | 16  | 16  | 0.47F |
| 1K6ax12dB | 1600 | 12dB | 25  | 16  | 0.47F |
| 1K6ax14dB | 1600 | 14dB | 30  | 14  | 0.33F |
| 1K6ax16dB | 1600 | 16dB | 40  | 12  | 0.33F |
| 1K6ax18dB | 1600 | 18dB | 50  | 12  | 0.22F |
| 1K6ax20dB | 1600 | 20dB | 75  | 12  | 0.22F |

Crossover 800

| Part Number | Xover | HF atten | R1 | R2 | C1 |
| --- | --- | --- | --- | --- | --- |
| 800ax06dB | 800 | 6dB | 8  | 30  | 3.3F |
| 800ax09dB | 800 | 9dB | 12  | 20  | F |
| 800ax12dB | 800 | 12dB | 20  | 14  | F |
| 800ax15dB | 800 | 15dB | 30  | 12  | F |
| 800ax18dB | 800 | 18dB | 50  | 12  | F |
| 800ax21dB | 800 | 21dB | 75  | 10  | F |

Crossover 600

| Part Number | Xover | HF atten | R1 | R2 | C1 |
| --- | --- | --- | --- | --- | --- |
| 600ax06dB | 800 | 6dB | 8  | 16  | F |
| 600ax09dB | 800 | 9dB | 12  | 10  | F |
| 600ax12dB | 800 | 12dB | 20  | 10  | F |
| 600ax15dB | 800 | 15dB | 30  | 8  | F |
| 600ax18dB | 800 | 18dB | 50  | 8  | F |
| 600ax21dB | 800 | 21dB | 75  | 8  | F |

Crossover 500

| Part Number | Xover | HF atten | R1 | R2 | C1 |
| --- | --- | --- | --- | --- | --- |
| 500ax06dB | 800 | 6dB | 6  | 16  | 6.6F |
| 500ax09dB | 800 | 9dB | 12  | 10  | F |
| 500ax12dB | 800 | 12dB | 20  | 10  | F |
| 500ax15dB | 800 | 15dB | 30  | 8  | F |
| 500ax18dB | 800 | 18dB | 50  | 8  | F |
| 500ax21dB | 800 | 21dB | 75  | 8  | F |

 Speakers

Crossover component values by frequency

| Crossover | L1 | C2 | C3 | L2 | C4 |
| --- | --- | --- | --- | --- | --- |
| 500a0xxdB | 1.5mH | F | F | 3.0mH | 33F |
| 600a0xxdB | 1.2mH | F | F | 2.25mH | F |
| 800a0xxdB | 1.2mH | F | F | 2.0mH | F |
| 1600a0xxdB | 0.6mH | F | F | 1.0mH | F |

Note:	Components L2 and C4 values shown for standard Butterworth models.  Component values for Pseudo First-Order models shown below.

 Speakers

Pseudo First-Order inductor values by woofer motor

1K6aLxxdB Crossover

| Woofer Motor | Crossover | Inductor L2 |
| --- | --- | --- |
| 2226H | 1K6aLxxdB | 0.7mH |
| 2227H | 1K6aLxxdB | 1.0mH |

Notes:

- Components R3, C4 and C5 are not used with woofers listed on this page.  In  all  other  respects,  the 1K6aLxxdB crossover is the same as the standard 1K6a0xxdB 1600Hz crossover.